# CIS2232 / CIS2250 Business Topic

This document will provide details on the specific business need that the web application in CIS2232 and mobile application in CIS2250 will be built to serve.  This is a custom topic that you think up.  For the topic that you come up with, you will be the business representative.  Another student will be the developer for this topic.  There are to be between 6-10 fields to be captured so the scope is limited.  Note the number of fields can be adjusted slightly if needed but to be kept reasonable.

As a part of the project, there must be a calculation involved.  This could be a cost, an average or some other accumulation based on several of the input fields but some type of calculation to be described.   It can be anything really but something that would take some manipulation / calculations based on the input fields.  You may make up some details to allow calculation/processing.

In groups of three – interview each other to find something that each person is interested in.  This can be related to a hobby that the student has, a prior job that they had, or a sport that they like.  Brainstorm the topic to come up with a little information system that can be used as a topic for second year courses.

# Project Group

| BA / Business Client |  |
| --- | --- |
| Developer | Kody |
| Project Manager / QA |  |

# Project Title

Aircraft Weight and Balance Sheet

# Project Base Color

Aviation navy - #1B3A5C

# Description (300 words)

This project will be used to record and check the weight and balance of a small aircraft before each flight. Every flight in a light aircraft begins with a weight and balance calculation. The pilot has to prove two things: that the loaded aircraft is under its maximum takeoff weight, and that its centre of gravity falls inside the envelope the manufacturer approved. An aircraft over gross weight will not climb as certified. One loaded outside its centre of gravity range can be unrecoverable in a stall even when it is under weight. The check is required before every flight and is normally done by hand on a paper form at the flight school desk. The system serves a flying club operating a single aircraft type, the Cessna 172S. One record is created per flight. The pilot enters the date, the tail number, their own name, the aircraft's empty weight and empty arm taken from its current weight and balance record, the weight in the front seats, the weight in the rear seats, the baggage weight, and the fuel loaded in US gallons. From those entries the system works out the loaded weight and the centre of gravity, compares both against the aircraft's limits, and returns a pass or fail with the reason for any failure. It also reports how much useful load is left and how much more fuel could be carried without going over gross weight, which is the question a pilot actually asks when a fourth passenger turns up at the hangar. Stored sheets give the club a history: which aircraft are habitually loaded near their limits, which flights were refused and why, and a record that the check was performed. Members can copy a previous sheet forward rather than re-entering the same aircraft data every time.

# Fields

| Name | Data Type | Description |
| --- | --- | --- |
| loadSheetId | int | Unique id for the load sheet //not user entered |
| flightDate | String | Date of the flight (yyyy-MM-dd) |
| tailNumber | String | Aircraft registration, e.g. CGABC |
| pilotName | String | Name of the pilot in command |
| emptyWeight | double | Empty weight of the aircraft in lb |
| emptyArm | double | Empty CG arm in inches aft of the datum |
| frontSeatWeight | double | Combined weight of pilot and front passenger in lb |
| rearSeatWeight | double | Combined weight of the rear passengers in lb |
| baggageWeight | double | Weight in the baggage area in lb |
| fuelLoaded | double | Usable fuel at takeoff in US gallons |
| totalWeight | double | Loaded weight of the aircraft in lb //calculated |
| centreOfGravity | double | Loaded CG in inches aft of the datum //calculated |
| loadStatus | String | PASS or FAIL against the aircraft limits //calculated |

# Calculation

Constants for the Cessna 172S, held in the application and not entered by the user: Station arms in inches aft of datum: front seats 37.0, rear seats 73.0, baggage 95.0, fuel 48.0 Avgas weighs 6.0 lb per US gallon Maximum gross takeoff weight 2550 lb Maximum baggage 120 lb Usable fuel capacity 53 US gallons Aft CG limit 47.3 in Forward CG limit 35.0 in at or below 1950 lb, sloping to 41.0 in at 2550 lb totalWeight calculation: fuelWeight = fuelLoaded * 6.0 totalWeight = emptyWeight + frontSeatWeight + rearSeatWeight + baggageWeight + fuelWeight centreOfGravity calculation: A moment is a weight multiplied by its arm, the distance from the datum. emptyMoment = emptyWeight * emptyArm frontMoment = frontSeatWeight * 37.0 rearMoment = rearSeatWeight * 73.0 baggageMoment = baggageWeight * 95.0 fuelMoment = fuelWeight * 48.0 totalMoment = emptyMoment + frontMoment + rearMoment + baggageMoment + fuelMoment centreOfGravity = totalMoment / totalWeight loadStatus calculation: forwardLimit = 35.0 when totalWeight is 1950 or less forwardLimit = 35.0 + (totalWeight - 1950) * 6.0 / 600 when totalWeight is over 1950 loadStatus is PASS only when all three of these hold: totalWeight is 2550 or less baggageWeight is 120 or less centreOfGravity is between forwardLimit and 47.3 Any failure is reported with the value, the limit and the amount over. Also reported: usefulLoadRemaining = 2550 - totalWeight maxAdditionalFuel = the smaller of usefulLoadRemaining / 6.0 and 53 - fuelLoaded, never below 0 Example calculation: emptyWeight 1680 at emptyArm 39.1, frontSeatWeight 340, rearSeatWeight 190, baggageWeight 45, fuelLoaded 40 fuelWeight = 40 * 6.0 = 240 totalWeight = 1680 + 340 + 190 + 45 + 240 = 2495 totalMoment = 65688 + 12580 + 13870 + 4275 + 11520 = 107933 centreOfGravity = 107933 / 2495 = 43.26 forwardLimit = 35.0 + (2495 - 1950) * 6.0 / 600 = 40.45 loadStatus = PASS, 43.26 is between 40.45 and 47.3 and the aircraft is under gross usefulLoadRemaining = 2550 - 2495 = 55 maxAdditionalFuel = 55 / 6.0 = 9.2 gallons

# Repository

Once the document is completed, move the details a repository that you create.  The developer creates the repository for the topic that they will be coding.  Add the other two project group members to the repository(or make public).  Add this document to the repository (Commit/Push).  The description is to be added to the readme file of the repository.